System verilog LAB Programs:

1.Write a SV code to demonstrate 2-state and 4-state data types

TESTBENCH

module state_demo_tb;

// Instantiate the DUT (Design Under Test)

state_demodut();

initial begin

// Record waveform for Riviera-PRO

$recordfile("waves.asdb");

$recordvars();

$display("Running testbench...");

#20;

$display("Simulation completed.");

$finish;

end

endmodule

RESULT

2. Write a SV code to demonstrate the difference between bit and byte.

module bit_byte_demo;

// Declare a single bit

bit my_bit;

// Declare a byte (8 bits)

byte my_byte;

initial begin

$display("=== bit vs byte demonstration ===");

// Assign values

my_bit = 1;

my_byte = 8'b1010_1100; // binary representation

$display("my_bit   = %b (size = %0d bit)", my_bit, $bits(my_bit));

$display("my_byte  = %b (size = %0d bits)", my_byte, $bits(my_byte));

// Show decimal equivalent of byte

$display("my_byte (decimal) = %0d", my_byte);

// Assigning a value greater than 1 to a bit will truncate

my_bit = 1'b1;

$display("\nAssigning 1 to bit: my_bit = %b", my_bit);

my_bit = 1'b0;

$display("Assigning 0 to bit: my_bit = %b", my_bit);

// Bit cannot hold multi-bit values (truncated to LSB)

my_bit = 8'b11110000;

$display("Assigning 8-bit value to bit (truncated): my_bit = %b", my_bit);

// Byte holds 8 bits

my_byte = 8'b00001111;

$display("Assigning 8-bit value to byte: my_byte = %b", my_byte);

end

endmodule

RESULT

3. Write a SV code to demonstrate the difference between int and integer.

module int_vs_integer_demo;

int    sv_int;

integer verilog_integer;

initial begin

// Assign positive and negative values

sv_int           = -32'd100;

verilog_integer  = -32'd200;

// Display the values

$display("SystemVerilog 'int' value      = %0d",sv_int);

$display("Verilog 'integer' value        = %0d", verilog_integer);

// Overflow behavior demonstration

sv_int           = 32'hFFFFFFFF;  // -1 in 32-bit signed

verilog_integer  = 32'hFFFFFFFF;  // -1 in 32-bitsigned

$display("After assigning 0xFFFFFFFF:");

$display("'int' as signed decimal       = %0d", sv_int);

$display("'integer' as signed decimal   = %0d",  verilog_integer);

end

endmodule   // Correct keyword to properly end the module

TEST BENCH

module int_vs_integer_demo;

int    sv_int;

integer verilog_integer;

initial begin

sv_int           = -32'd100;

verilog_integer  = -32'd200;

$display("SystemVerilog 'int' value      = %0d", sv_int);

$display("Verilog 'integer' value        = %0d", verilog_integer);

sv_int           = 32'hFFFFFFFF;

verilog_integer  = 32'hFFFFFFFF;

$display("After assigning 0xFFFFFFFF:");

$display("'int' as signed decimal       = %0d", sv_int);

$display("'integer' as signed decimal   = %0d", verilog_integer);

end

endmodule

RESULT

2.(a)Write a SV code to demonstrate for and foreach loops for multidimensional arrays

module array_ops;

// Declare a 2D static array of size 3x4

int matrix[3][4];

// Task to initialize array using 'for' loop

task initialize_with_for();

for (int i = 0; i< 3; i++) begin

for (int j = 0; j < 4; j++) begin

matrix[i][j] = i * 10 + j;

end

end

endtask

// Task to display array using 'foreach' loop

task display_with_foreach(string msg);

$display("=== %s ===", msg);

foreach (matrix[i, j]) begin

$display("matrix[%0d][%0d] = %0d", i, j, matrix[i][j]);

end

endtask

// Task to modify array using 'foreach' loop (e.g., double values)

task modify_with_foreach();

foreach (matrix[i, j]) begin

matrix[i][j] *= 2;

end

endtask

endmodule

TEST BENCH

`timescale 1ns / 1ps

module testbench;

// Instantiate the design module

array_opsuut();  // uut = unit under test

initial begin

// Step 1: Initialize array with 'for' loop

uut.initialize_with_for();

// Step 2: Display initial values using 'foreach'

uut.display_with_foreach("Initial values (after for-loop)");

// Step 3: Modify values using 'foreach'

uut.modify_with_foreach();

// Step 4: Display modified values

uut.display_with_foreach("Modified values (after foreach)");

$finish;

End

Endmodule

RESULT

2.(b) Write a SV code to demonstrate packed and unpacked array

module packed_unpacked_example;

// Packed array: 4 elements of 8-bit logic, packed into a single vector [3:0][7:0]

logic [7:0] packed_array [3:0]; // Equivalent to: array of 4 bytes packed together

// Unpacked array: 4 separate 8-bit vectors

logic [3:0] unpacked_array [7:0]; // 8 elements, each is a 4-bit wide logic vector

initial begin

// Initialize packed array

$display("=== Packed Array Initialization ===");

for (int i = 0; i< 4; i++) begin

packed_array[i] = i * 10;

$display("packed_array[%0d] = %0d", i, packed_array[i]);

end

// Initialize unpacked array

$display("\n=== Unpacked Array Initialization ===");

for (int i = 0; i< 8; i++) begin

unpacked_array[i] = i;

$display("unpacked_array[%0d] = %0d", i, unpacked_array[i]);

end

// Show bit-level of packed array

$display("\n=== Bit-level Representation of Packed Array Elements ===");

for (int i = 0; i< 4; i++) begin

$display("packed_array[%0d] = %b", i, packed_array[i]);

end

// Show bit-level of unpacked array

$display("\n=== Bit-level Representation of Unpacked Array Elements ===");

for (int i = 0; i< 8; i++) begin

$display("unpacked_array[%0d] = %b", i, unpacked_array[i]);

end

$finish;

end

endmodule

TEST BENCH

`timescale 1ns/1ps

module testbench;

// Instantiate DUT here and your test logic

initial begin

$display("Test started");

#10;

$display("Test finished");

$finish;

end

endmodule

RESULT

3(a) Write a SV code to implement a module to store and display a list of student grades using a dynamic array.

// Dynamic array to store grades

Module_student_grade

int grades[];  // dynamic array of integers

// Task to add grades

task add_grades(input int grade_list[], input int count);

grades = new[count];  // allocate memory

foreach (grade_list[i]) begin

grades[i] = grade_list[i];

end

endtask

// Task to display grades

task display_grades();

$display("List of Student Grades:");

foreach (grades[i]) begin

$display("Student %0d: Grade = %0d", i+1, grades[i]);

end

endtask

// Initial block to simulate behavior

initial begin

int temp_grades[] = '{85, 90, 78, 92, 88};  // Sample grades

int num_students = temp_grades.size();

// Add and display grades

add_grades(temp_grades, num_students);

display_grades();

end

endmodule

TEST BENCH

module tb_student_grades;

// Instantiate the DUT (Device Under Test)

student_gradesdut();  // No ports, simple instantiation

initial begin

$display("Starting Student Grades Testbench...\n");

// Wait for the DUT to complete (simulation happens in its initial block)

#10;

$display("\nTestbench completed.");

$finish;

end

endmodule

// Code your design here

module StudentGrades;

int grades[];  // dynamic array

// Initialize dynamic array

initial begin

grades = new[0];

end

// Task to add a grade

task add_grade(input int grade);

endtask

// Task to display all grades

task display_grades();

if (grades.size() == 0) begin

$display("No grades available.");

end else begin

$display("Student Grades:");

for (int i = 0; i < grades.size(); i++) begin

$display("Grade[%0d] = %0d", i, grades[i]);

end

end

endtask

endmodule

module testbench;

StudentGrades dut();

initial begin

dut.add_grade(85);

dut.add_grade(90);

dut.add_grade(78);

dut.display_grades();

$finish;

end

endmodule

3.b) ) Develop a module that maps product names to their prices using associative array

module ProductPriceMap;

// Associative array mapping product names (string) to prices (real)

// You can use int or real for price depending on requirement.

real prices[string];

// Task to add or update a product price

task add_product(string product_name, real price);

prices[product_name] = price;

endtask

// Task to get price of a product

function real get_price(string product_name);

if (prices.exists(product_name)) begin

return prices[product_name];

end else begin

$display("Product \"%s\" not found!", product_name);

return -1.0;  // Indicate not found

end

endfunction

// Task to display all products and their prices

task display_all();

if (prices.num() == 0) begin

$display("No products available.");

end else begin

$display("Product Price List:");

foreach (prices[name]) begin

$display("%s : $%0.2f", name, prices[name]);

end

end

endtask

endmodule

Testbench

// Testbench to demonstrate

module testbench;

ProductPriceMap ppm();

initial begin

ppm.add_product("Apple", 0.99);

ppm.add_product("Banana", 0.59);

ppm.add_product("Orange", 1.25);

ppm.display_all();

$display("Price of Banana: $%0.2f", ppm.get_price("Banana"));

$display("Price of Mango: $%0.2f", ppm.get_price("Mango"));

$finish;

end

endmodule

RESULT

3.c) Write a SV code to demonstrate all the Queue operations.

module QueueOperations;

int queue[$];  // queue declaration

int front_element;  // declare variables at module scope

int back_element;

initial begin

$display("Queue operations demonstration:");

// 1. Enqueue elements (push_back)

queue.push_back(10);

queue.push_back(20);

queue.push_back(30);

$display("After push_back 10,20,30: %p", queue);

// 2. Dequeue element from front (pop_front)

front_element = queue.pop_front();

$display("After pop_front (removed %0d): %p", front_element, queue);

// 3. Enqueue element at front (push_front)

queue.push_front(5);

$display("After push_front 5: %p", queue);

// 4. Dequeue element from back (pop_back)

back_element = queue.pop_back();

$display("After pop_back (removed %0d): %p", back_element, queue);

// 5. Access element by index

$display("Element at index 0: %0d", queue[0]);

// 6. Insert element at specific index

queue.insert(1, 15); // Insert 15 at index 1

$display("After insert 15 at index 1: %p", queue);

// 7. Delete element at specific index

queue.delete(0); // Delete element at index 0

$display("After delete element at index 0: %p", queue);

// 8. Size of queue

$display("Queue size: %0d", queue.size());

// 9. Check if queue is empty

$display("Is queue empty? %s", (queue.size() == 0) ? "Yes" : "No");

// 10. Clear the queue

queue.delete();

$display("After clearing queue: %p (size=%0d)", queue, queue.size());

$finish;

end

endmodule

RESULT

4.a) Write a SV code to demonstrate break and continue statements

module BreakContinueDemo;

initial begin

$display("Demonstrate 'continue':");

// Loop from 1 to 10

for (int i = 1; i <= 10; i++) begin

if (i % 2 == 0) begin

// Skip even numbers

continue;

end

$display("Odd number: %0d", i);

end

$display("\nDemonstrate 'break':");

// Loop from 1 to 10

for (int i = 1; i <= 10; i++) begin

if (i > 5) begin

// Exit loop if i > 5

break;

end

$display("Value: %0d", i);

end

$finish;

end

endmodule

RESULT

4.b) Develop a module to demonstrate Tasks, Functions and Void Function.

module TasksFunctionsDemo;

// Declare variables at module scope

integer val;

integer val_sq;

// Task: Can have input/output, can contain delays

task print_message(input string msg);

$display("Task: %s", msg);

endtask

// Function: Returns integer, no delays allowed

function integer square(input integer x);

square = x * x;

endfunction

// Void function: No return value

function void display_sum(input integer a, input integer b);

integer sum;

sum = a + b;

$display("Void function: Sum of %0d and %0d is %0d", a, b, sum);

endfunction

initial begin

// Call task

print_message("Hello from task");

// Initialize variables declared at module level

val = 5;

val_sq = square(val);

$display("Function: Square of %0d is %0d", val, val_sq);

// Call void function

display_sum(10, 20);

$finish;

end

endmodule

RESULT

5.a) Write a SV code to demonstrate rand and randc with random class.

// Code your design here

class RandomExample;

// 'rand' — normal random variable (values can repeat)

rand bit [3:0] rand_val;

// 'randc' — cyclic random variable (values do not repeat until all used)

randc bit [3:0] randc_val;

// Optional constraints (limit value range)

constraint val_range {

rand_val inside {[0:9]};   // restrict rand_val to 0–9

randc_val inside {[0:9]};  // restrict randc_val to 0–9

}

// Display results

function void display();

$display("rand_val = %0d, randc_val = %0d", rand_val, randc_val);

endfunction

endclass

Testbench

module tb;

RandomExample obj;

initial begin

obj = new();

$display("=== Demonstrating rand and randc ===");

$display("  rand → may repeat values");

$display("  randc → cycles through values without repetition\n");

// Run randomization multiple times

repeat (15) begin

if (obj.randomize()) begin

obj.display();

end else begin

$display("Randomization failed!");

end

end

$finish;

end

endmodule

Result

5.b) Write a constraint for removing duplicate elements from queue.

class UniqueQueueExample;

// Declare a random queue of integers

rand int q[$];

// Size constraint

constraint size_c {

q.size() == 5;     // Queue will always have 5 elements

}

⁸

// Uniqueness constraint

constraint unique_c {

unique { q };      // Ensures all queue elements are unique

}

// Value range constraint (optional)

constraint range_c {

foreach (q[i]) q[i] inside {[1:10]};   // Each element between 1–10

}

// Display method

function void display();

$display("Unique queue = %p", q);

endfunction

endclass

Test bench

module tb;

UniqueQueueExample obj;

initial begin

obj = new();

$display("=== Demonstrating constraint to remove duplicates from queue ===\n");

// Try randomizing multiple times

repeat (5) begin

if (obj.randomize()) begin

obj.display();

end else begin

$display("Randomization failed!");

end

end

$finish;

end

endmodule

Result

5.c) ) Write a constraint to generate even numbers between 10 to 30.

class EvenNumberGenerator;

rand int num;  // Random variable

// Constraint block to restrict range and ensure even numbers

constraint even_range {

num inside {[10:30]};   // Range between 10 and 30

num % 2 == 0;           // Must be even

}

endclass

module even_numbers_constraint;

EvenNumberGenerator eng = new();

initial begin

$display("Random even numbers between 10 and 30:");

repeat (10) begin

assert(eng.randomize()) else $error("Randomization failed!");

$display("%0d", eng.num);

end

end

endmodule

RESULT

6.a) Write a constraint to arrange the initial numbers in ascending order and remaining numbers in descending order.

// Code your design here

class OrderArray;

rand int arr[];  // dynamic array

// Constraint: size of the array and ordering

constraint c_arr_size { arr.size() == 10; }  // example size = 10

// Constraint to arrange:

// - first half ascending

// - second half descending

constraint c_order {

foreach (arr[i]) {

// Ascending order for first half

if (i < arr.size()/2 - 1)

arr[i] < arr[i+1];

// Descending order for second half

if (i >= arr.size()/2 && i < arr.size()-1)

arr[i] > arr[i+1];

}

}

endclass

Testbench

module tb_order_array;

OrderArray obj = new();

initial begin

assert(obj.randomize()) else $error("Randomization failed!");

$display("Array with first half ascending and second half descending:");

foreach (obj.arr[i])

$write("%0d ", obj.arr[i]);

$display("");  // newline

end

endmodule

Result

6.b) Write a constraint to generate a sequence of 9 99 999 9999 99999.

class SequenceGen;

rand int arr[];

// fix the size to 5

constraint size_c { arr.size() == 5; }

// enforce the sequence: arr[0] == 9; arr[i] == arr[i-1]*10 + 9

constraint pattern_c {

arr[0] == 9;

foreach (arr[i]) if (i > 0) arr[i] == arr[i-1] * 10 + 9;

}

endclass

Testbench

module tb_sequence_gen;

SequenceGen sg = new();

initial begin

// randomize the array (but constraints force exact values)

if (!sg.randomize()) begin

$error("randomize() failed");

$finish;

end

$display("Constrained sequence (SystemVerilog):");

foreach (sg.arr[i]) $display("%0d: %0d", i, sg.arr[i]);

$finish;

end

endmodule

Result

7.a) Write a constraint to generate:

- A queue with unique value.

class unique_queue_gen;

rand int q[];              // Declare a random queue of integers

rand int size;             // Random queue size

constraint c_size {

size inside {[5:10]};    // Queue size between 5 and 10

}

constraint c_unique {

q.size() == size;        // Queue must match the chosen size

unique { q };            // All elements in the queue must be unique

}

// Optional: limit the range of values

constraint c_values {

foreach (q[i]) q[i] inside {[0:50]};

}

function void display();

$display("Unique Queue (size=%0d): %p", q.size(), q);

endfunction

endclass

module test;

initial begin

unique_queue_gen gen = new();

// Try multiple randomizations

repeat (5) begin

if (gen.randomize())

gen.display();

else

$display("Randomization failed!");

end

end

endmodule

RESULT

- Store a even number at the index of the odd value.

class even_at_odd_index;

rand int arr[];        // random array

rand int size;         // size of the array

constraint c_size {

size inside {[5:10]};      // array size between 5 and 10

arr.size() == size;

}

// Constraint: Even number at the index equal to an odd value

constraint c_even_at_odd_index {

foreach (arr[i]) {

// If index 'i' is odd, arr[i] must be even

if (i % 2 == 1)

arr[i] % 2 == 0;

else

arr[i] % 2 == 1;  // (optional) make even indices store odd numbers

}

}

// Limit the range of the values (optional)

constraint c_range {

foreach (arr[i])

arr[i] inside {[0:50]};

}

function void display();

$display("Array (size=%0d): %p", arr.size(), arr);

endfunction

endclass

module test;

initial begin

even_at_odd_index obj = new();

repeat (5) begin

if (obj.randomize())

obj.display();

else

$display("Randomization failed!");

end

end

endmodule

RESULT

- Store a odd number at the index of the even value

class odd_at_even_index;

rand int arr[];     // Random array

rand int size;      // Size of array

// Constraint: array size between 5 and 10

constraint c_size {

size inside {[5:10]};

arr.size() == size;

}

// Main constraint:

// Store an odd number at every even index

constraint c_odd_at_even_index {

foreach (arr[i]) {

if (i % 2 == 0)

arr[i] % 2 == 1;   // odd value at even index

else

arr[i] % 2 == 0;   // optional: even value at odd index

}

}

// Optional: Limit the value range

constraint c_range {

foreach (arr[i])

arr[i] inside {[0:50]};

}

// Display function

function void display();

$display("Array (size=%0d): %p", arr.size(), arr);

endfunction

endclass

module test;

initial begin

odd_at_even_index obj = new();

// Generate multiple examples

repeat (5) begin

if (obj.randomize())

obj.display();

else

$display("Randomization failed!");

end

end

endmodule

RESULT

7.b) Write a constraint to generate a pattern of 000111222333444555

class pattern_gen;

rand int arr[];     // dynamic array to hold the pattern

// Constraint to generate the pattern 000111222333444555

constraint c_pattern {

arr.size() == 18; // total 18 elements (6 groups × 3 repeats)

foreach (arr[i]) {

arr[i] == (i / 3); // every 3 elements have the same number

}

}

// Display

function void display();

$display("Generated Pattern: %p", arr);

endfunction

endclass

module test;

initial begin

pattern_gen obj = new();

if (obj.randomize())

obj.display();

else

$display("Randomization failed!");

end

endmodule

RESULT

8.a) Demonstrate fork-join, for-join-any, fork-join-none using suitable SV code.

module fork_join_demo;

// Simple tasks to simulate concurrent work

task automatic task_A();

$display("[%0t] Task A started", $time);

#10;

$display("[%0t] Task A finished", $time);

endtask

task automatic task_B();

$display("[%0t] Task B started", $time);

#20;

$display("[%0t] Task B finished", $time);

endtask

task automatic task_C();

$display("[%0t] Task C started", $time);

#30;

$display("[%0t] Task C finished", $time);

endtask

// ----------------------------------------------------------

// fork...join  → waits for all threads to finish

// ----------------------------------------------------------

initial begin

$display("\n=== Example 1: fork...join ===");

fork

task_A();

task_B();

task_C();

join

$display("[%0t] All tasks finished → continuing (fork-join)\n", $time);

end

// ----------------------------------------------------------

// fork...join_any → waits until *any one* thread finishes

// ----------------------------------------------------------

initial begin

#50; // delay to separate examples

$display("\n=== Example 2: fork...join_any ===");

fork

task_A();

task_B();

task_C();

join_any

$display("[%0t] One of the tasks finished → continuing (fork-join_any)\n", $time);

#40; // let others finish for clarity

end

// ----------------------------------------------------------

// fork...join_none → does not wait at all

// ----------------------------------------------------------

initial begin

#120; // delay to separate examples

$display("\n=== Example 3: fork...join_none ===");

fork

task_A();

task_B();

task_C();

join_none

$display("[%0t] Main thread continues immediately (fork-join_none)\n", $time);

#40; // wait to observe other threads

$display("[%0t] End of simulation\n", $time);

end

endmodule

RESULT

8.b) Develop module of the mailbox connecting two transactors.

`timescale 1ns/1ps

// Simple transaction object

class txn;

rand int unsigned id;

rand bit [31:0] data;

function new(int unsigned id = 0, int unsigned data = 0);

this.id   = id;

this.data = data;

endfunction

function string to_string();

return $sformatf("{id=%0d data=0x%0h}", id, data);

endfunction

endclass

// Producer transactor: generates transactions and puts them onto the mailbox

class producer_tx;

mailbox #(txn) mb;         // typed mailbox handle

int unsigned num_to_send;

int unsigned delay_ns;     // optional inter-transaction delay (ns)

function new(mailbox #(txn) mb, int unsigned n = 10, int unsigned d = 10);

this.mb = mb;

this.num_to_send = n;

this.delay_ns = d;

endfunction

task run();

txn t;

for (int unsigned i = 1; i <= num_to_send; i++) begin

// create a transaction and randomize its fields (optional)

t = new(i, $urandom());

if (!t.randomize()) begin

// ignore randomize failure; it's optional

end

// blocking put: if mailbox full, this will wait

$display("[%0t] Producer: putting txn %s", $time, t.to_string());

mb.put(t);

// optional delay between transactions to emulate timing

#delay_ns;

end

// send a null/poison-pill to indicate end of stream (consumer should detect)

$display("[%0t] Producer: sending end-of-stream marker", $time);

mb.put(null);

endtask

endclass

// Consumer transactor: gets transactions from mailbox and processes them

class consumer_tx;

mailbox #(txn) mb;

int unsigned processed = 0;

function new(mailbox #(txn) mb);

this.mb = mb;

endfunction

task run();

txn t;

forever begin

// blocking get: waits until something is available

mb.get(t);

// check for "end of stream" marker (null pointer)

if (t == null) begin

$display("[%0t] Consumer: received end-of-stream marker, processed=%0d. Exiting.",

$time, processed);

break;

end

// process the transaction

processed++;

$display("[%0t] Consumer: got txn %s (processed=%0d)", $time, t.to_string(), processed);

// emulate processing time

#5;

end

endtask

endclass

Testbench

module tb_mailbox;

// typed mailbox of txn objects; size parameter is optional (0=unbounded),

// here we create a mailbox with depth 4 for demonstration (bounded)

mailbox #(txn) mb = new(4);

// instances

producer_tx prod;

consumer_tx cons;

initial begin

$display("=== Mailbox transactors demo ===");

// create transactors

prod = new(mb, 12, 7);  // send 12 txns, 7ns gap

cons = new(mb);

// launch producer and consumer concurrently

fork

prod.run();

cons.run();

join_any

// wait for both to finish: we expect consumer to exit after reading null

// However join_any returns when first finishes; ensure both are done:

// Use a small loop to poll mailbox/processed state or simply wait long enough.

// For clarity we will wait until consumer posts the end-of-stream message and then finish.

// A robust environment would coordinate using events or a join of the processes.

// Give enough time for final prints, then finish simulation

#200;

$display("=== Simulation complete ===");

$finish;

end

endmodule

Output

DEMO EXPERIMENT

9) Demonstrate complete SV design that incorporates the entire SystemVerilog environment for simple digital circuit.

interface intf(input bit clk);

logic a, b, y;   // simple AND gate DUT signals

endinterface

module dut(intf i);

assign i.y = i.a & i.b;

endmodule

class transaction;

rand bit a, b;

bit  y;           // expected output

function void display(string tag);

$display("[%s] a=%0b b=%0b y=%0b", tag, a, b, y);

endfunction

endclass

class generator;

mailbox gen2drv;

int num;

function new(mailbox m, int n);

gen2drv = m;

num     = n;

endfunction

task run();

transaction t;

repeat(num) begin

t = new();

assert(t.randomize());

t.display("GEN");

gen2drv.put(t);

end

endtask

endclass

class driver;

virtual intf vif;

mailbox gen2drv, drv2mon;

function new(virtual intf v, mailbox m1, mailbox m2);

vif      = v;

gen2drv  = m1;

drv2mon  = m2;

endfunction

task run();

transaction t;

forever begin

gen2drv.get(t);     // safe → always object (no null)

t.display("DRV");

vif.a = t.a;

vif.b = t.b;

#10;

drv2mon.put(t);

end

endtask

endclass

class monitor;

virtual intf vif;

mailbox drv2mon;

function new(virtual intf v, mailbox m);

vif      = v;

drv2mon  = m;

endfunction

task run();

transaction t;

forever begin

drv2mon.get(t);   // safe → driver always sends

t.y = vif.y;

t.display("MON");

end

endtask

endclass

class environment;

generator gen;

driver    drv;

monitor   mon;

mailbox gen2drv = new();

mailbox drv2mon = new();

virtual intf vif;

function new(virtual intf v);

vif = v;

gen = new(gen2drv, 5);

drv = new(vif, gen2drv, drv2mon);

mon = new(vif, drv2mon);

endfunction

task run();

fork

gen.run();

drv.run();

mon.run();

join_any

$display("ENV DONE");

endtask

endclass

Testbench

module tb;

bit clk = 0;

always #5 clk = ~clk;

intf i(clk);

dut d(i);

environment env;

initial begin

env = new(i);     // IMPORTANT → avoids NULL POINTER

env.run();

#100 $finish;

end

endmodule

Output

10) Demonstrate simple protocol through the SV environment.

// Code your design here

interface proto_if(input logic clk);

logic req;

logic ack;

endinterface

module proto_dut(

input  logic clk,

input  logic rst,

input  logic req,

output logic ack

);

int delay_cnt = 0;

always_ff @(posedge clk or posedge rst) begin

if (rst) begin

ack <= 0;

delay_cnt <= 0;

end else begin

if (req) begin

delay_cnt <= 2;          // respond in 2 cycles

end else if (delay_cnt > 0) begin

delay_cnt <= delay_cnt - 1;

end

ack <= (delay_cnt == 1);

end

end

endmodule

module driver(

input  logic clk,

input  logic rst,

output logic req

);

initial begin

req = 0;

@(negedge rst);

repeat (5) begin

@(posedge clk);

req <= $urandom_range(0,1);

$display("[DRV] Time=%0t req=%0b", $time, req);

end

end

endmodule

module monitor(

input logic clk,

input logic req,

input logic ack

);

always @(posedge clk) begin

$display("[MON] Time=%0t req=%0b ack=%0b", $time, req, ack);

end

endmodule

Testbench

module tb;

logic clk = 0;

logic rst = 1;

always #5 clk = ~clk;

proto_if pif(clk);

proto_dut dut(

.clk(clk),

.rst(rst),

.req(pif.req),

.ack(pif.ack)

);

driver drv(

.clk(clk),

.rst(rst),

.req(pif.req)

);

monitor mon(

.clk(clk),

.req(pif.req),

.ack(pif.ack)

);

initial begin

#15 rst = 0;

#200 $finish;

end

endmodule

Output